EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 17/02/2025 17:42:15 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.18675 | 1 | 18.20 | 44.00 | -25.80 | Vertical | 9.91 |
| 26.63875 | 2 | 5.87 | 28.00 | -22.13 | Vertical | 10.30 |

Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.26775 | 1 | 33.23 | 51.00 | -17.77 | Vertical | 9.92 |
| 27.48475 | 2 | 22.64 | 45.00 | -22.36 | Vertical | 10.29 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.26775 | 1 | 31.21 | 51.00 | -19.79 | Vertical | 9.92 |
| 27.48475 | 2 | 16.95 | 45.00 | -28.05 | Vertical | 10.29 |

Peak/Lim.Peak (2)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.26775 | 1 | 33.23 | 64.00 | -30.77 | Vertical | 9.92 |
| 27.48475 | 2 | 22.64 | 58.00 | -35.36 | Vertical | 10.29 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz |
| --- | --- |

| name | 100kHz-10MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 10MHz-30MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 17/02/2025 18:41:03 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.91025 | 1 | -1.24 | 4.00 | -5.24 | Vertical | 20.75 |
| 1559.32825 | 2 | -2.25 | 4.00 | -6.25 | Horizontal | 21.56 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 17/02/2025 20:19:07 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.91025 | 1 | -0.38 | 4.00 | -4.38 | Vertical | 20.75 |
| 1599.8755 | 2 | -2.01 | 4.00 | -6.01 | Horizontal | 21.58 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 18:47:17 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.9 | 1 | 12.07 | 18.00 | -5.93 | Vertical | -14.22 |
| 87.03 | 2 | 6.63 | 12.00 | -5.37 | Horizontal | -18.78 |
| 432.02 | 3 | 7.43 | 18.00 | -10.57 | Vertical | -10.22 |
| 432.5 | 4 | 3.62 | 18.00 | -14.38 | Horizontal | -10.19 |
| 1971.08 | 5 | 14.72 | 30.00 | -15.28 | Vertical | 6.11 |
| 1972.61 | 6 | 14.56 | 30.00 | -15.44 | Horizontal | 6.13 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.66 | 1 | 17.69 | 25.00 | -7.31 | Vertical | -14.23 |
| 164.34 | 2 | 17.19 | 25.00 | -7.81 | Horizontal | -14.61 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.02 | 3 | 20.87 | 25.00 | -4.13 | Vertical | -10.22 |
| 434 | 4 | 17.97 | 25.00 | -7.03 | Horizontal | -10.11 |
| 1958.21 | 5 | 28.71 | 43.00 | -14.29 | Vertical | 6.00 |
| 1979.27 | 6 | 28.81 | 43.00 | -14.19 | Horizontal | 6.18 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 18:48:46 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 175.5 | 1 | 19.19 | 22.00 | -2.81 | Vertical | -14.20 |
| 174 | 2 | 17.51 | 22.00 | -4.49 | Horizontal | -14.25 |
| 514.5 | 3 | 31.61 | 28.00 | 3.61 | Vertical | -8.37 |
| 244.75 | 4 | 23.61 | 22.00 | 1.61 | Horizontal | -14.50 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 173.5 | 1 | 30.59 | 32.00 | -1.41 | Vertical | -14.27 |
| 173.5 | 2 | 29.14 | 32.00 | -2.86 | Horizontal | -14.27 |
| 239 | 3 | 35.67 | 32.00 | 3.67 | Vertical | -14.34 |
| 245 | 4 | 35.27 | 32.00 | 3.27 | Horizontal | -14.52 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of harness RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 14:37:39 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 99.51 | 1 | 17.78 | 12.00 | 5.78 | Vertical | -18.10 |
| 142.2 | 2 | 10.62 | 18.00 | -7.38 | Horizontal | -15.70 |
| 918.68 | 3 | 22.49 | 30.00 | -7.51 | Vertical | -2.65 |
| 432.14 | 4 | 5.61 | 18.00 | -12.39 | Horizontal | -10.21 |
| 1972.1 | 5 | 14.63 | 30.00 | -15.37 | Vertical | 6.12 |
| 1972.55 | 6 | 14.67 | 30.00 | -15.33 | Horizontal | 6.13 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.24 | 1 | 24.76 | 25.00 | -0.24 | Vertical | -14.24 |
| 142.62 | 2 | 16.29 | 25.00 | -8.71 | Horizontal | -15.68 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.56 | 3 | 22.39 | 25.00 | -2.61 | Vertical | -10.19 |
| 434.63 | 4 | 19.54 | 25.00 | -5.46 | Horizontal | -10.11 |
| 1980.71 | 5 | 29.11 | 43.00 | -13.89 | Vertical | 6.20 |
| 1987.31 | 6 | 29.03 | 43.00 | -13.97 | Horizontal | 6.25 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 15:51:43 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 171 | 1 | 12.76 | 22.00 | -9.24 | Horizontal | -14.36 |
| 174.25 | 2 | 25.58 | 22.00 | 3.58 | Vertical | -14.24 |
| 244.5 | 3 | 31.62 | 22.00 | 9.62 | Vertical | -14.49 |
| 245 | 4 | 25.01 | 22.00 | 3.01 | Horizontal | -14.52 |
| 2137.5 | 5 | 27.80 | 36.00 | -8.20 | Vertical | 8.85 |
| 2467.25 | 6 | 26.99 | 36.00 | -9.01 | Horizontal | 9.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 172 | 1 | 25.33 | 32.00 | -6.67 | Horizontal | -14.32 |
| 175 | 2 | 35.85 | 32.00 | 3.85 | Vertical | -14.22 |
| 244.25 | 3 | 43.09 | 32.00 | 11.09 | Vertical | -14.48 |
| 244.5 | 4 | 36.94 | 32.00 | 4.94 | Horizontal | -14.49 |
| 2411.25 | 5 | 41.64 | 46.00 | -4.36 | Vertical | 9.79 |
| 2493.75 | 6 | 41.40 | 46.00 | -4.60 | Horizontal | 9.78 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 09:32:15 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.9125 | 1 | -0.93 | 4.00 | -4.93 | Vertical | 20.75 |
| 1599.8755 | 2 | -1.32 | 4.00 | -5.32 | Horizontal | 21.58 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 12:42:02 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Y, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 99.75 | 1 | 23.95 | 12.00 | 11.95 | Vertical | -15.57 |
| 90.78 | 2 | 22.54 | 12.00 | 10.54 | Horizontal | -15.97 |
| 432.95 | 3 | 6.20 | 18.00 | -11.80 | Vertical | -10.17 |
| 432.02 | 4 | 6.42 | 18.00 | -11.58 | Horizontal | -10.22 |
| 1976.39 | 5 | 14.74 | 30.00 | -15.26 | Vertical | 6.16 |
| 1977.2 | 6 | 14.69 | 30.00 | -15.31 | Horizontal | 6.17 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 144.42 | 1 | 29.20 | 25.00 | 4.20 | Vertical | -13.11 |
| 86.97 | 2 | 28.53 | 25.00 | 3.53 | Horizontal | -16.23 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 433.49 | 3 | 20.25 | 25.00 | -4.75 | Vertical | -10.14 |
| 433.13 | 4 | 19.61 | 25.00 | -5.39 | Horizontal | -10.16 |
| 1972.64 | 5 | 28.99 | 43.00 | -14.01 | Vertical | 6.13 |
| 1960.91 | 6 | 28.95 | 43.00 | -14.05 | Horizontal | 6.03 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 12:45:15 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Y, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 185.75 | 1 | 29.27 | 22.00 | 7.27 | Vertical | -11.29 |
| 171.25 | 2 | 26.16 | 22.00 | 4.16 | Horizontal | -11.90 |
| 238.75 | 3 | 30.57 | 22.00 | 8.57 | Vertical | -14.33 |
| 241.75 | 4 | 20.55 | 22.00 | -1.45 | Horizontal | -14.47 |
| 2137.5 | 5 | 29.11 | 36.00 | -6.89 | Vertical | 8.85 |
| 2137.5 | 6 | 27.92 | 36.00 | -8.08 | Horizontal | 8.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 186 | 1 | 39.08 | 32.00 | 7.08 | Vertical | -11.27 |
| 174.25 | 2 | 36.73 | 32.00 | 4.73 | Horizontal | -11.80 |
| 239.25 | 3 | 43.76 | 32.00 | 11.76 | Vertical | -14.35 |
| 240.75 | 4 | 33.21 | 32.00 | 1.21 | Horizontal | -14.42 |
| 2490.25 | 5 | 41.51 | 46.00 | -4.49 | Vertical | 9.79 |
| 2471 | 6 | 41.19 | 46.00 | -4.81 | Horizontal | 9.84 |

Test parameters

| Name test: | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 09:39:15 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.91025 | 1 | 2.20 | 4.00 | -1.80 | Vertical | 20.75 |
| 1175.908 | 2 | 0.15 | 4.00 | -3.85 | Horizontal | 20.75 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 12:57:45 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 90.39 | 1 | 21.49 | 12.00 | 9.49 | Vertical | -15.99 |
| 100.14 | 2 | 17.73 | 12.00 | 5.73 | Horizontal | -15.55 |
| 469.46 | 3 | 16.89 | 22.00 | -5.11 | Vertical | -8.71 |
| 432.26 | 4 | 10.34 | 18.00 | -7.66 | Horizontal | -10.21 |
| 1984.85 | 5 | 14.57 | 30.00 | -15.43 | Vertical | 6.23 |
| 1975.91 | 6 | 14.54 | 30.00 | -15.46 | Horizontal | 6.16 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 87.39 | 1 | 27.43 | 25.00 | 2.43 | Vertical | -16.20 |
| 142.89 | 2 | 24.00 | 25.00 | -1.00 | Horizontal | -13.19 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.5 | 3 | 24.08 | 25.00 | -0.92 | Vertical | -10.19 |
| 432.44 | 4 | 23.82 | 25.00 | -1.18 | Horizontal | -10.20 |
| 1984.07 | 5 | 28.92 | 43.00 | -14.08 | Vertical | 6.23 |
| 1993.76 | 6 | 29.18 | 43.00 | -13.82 | Horizontal | 6.31 |

Test parameters

| Name test: | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-01_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-01 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 13:08:40 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 172 | 1 | 28.64 | 22.00 | 6.64 | Vertical | -11.87 |
| 174.5 | 2 | 24.05 | 22.00 | 2.05 | Horizontal | -11.79 |
| 243.75 | 3 | 29.71 | 22.00 | 7.71 | Vertical | -14.47 |
| 514.5 | 4 | 33.16 | 28.00 | 5.16 | Horizontal | -8.37 |
| 2137.5 | 5 | 28.49 | 36.00 | -7.51 | Vertical | 8.85 |
| 2467.5 | 6 | 26.99 | 36.00 | -9.01 | Horizontal | 9.84 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 171.75 | 1 | 38.48 | 32.00 | 6.48 | Vertical | -11.88 |
| 173.5 | 2 | 34.67 | 32.00 | 2.67 | Horizontal | -11.82 |
| 242.5 | 3 | 41.62 | 32.00 | 9.62 | Vertical | -14.48 |
| 241.25 | 4 | 35.70 | 32.00 | 3.70 | Horizontal | -14.44 |
| 2446 | 5 | 41.81 | 46.00 | -4.19 | Vertical | 9.88 |
| 2445.75 | 6 | 41.50 | 46.00 | -4.50 | Horizontal | 9.88 |

Test parameters

| Name test: | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-01 OFF_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |