EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 17/02/2025 17:08:38 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | BGN ON, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.19125 | 1 | 18.29 | 44.00 | -25.71 | Vertical | 9.91 |
| 27.3475 | 2 | 6.17 | 28.00 | -21.83 | Vertical | 10.29 |

Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.162 | 1 | 33.19 | 51.00 | -17.81 | Vertical | 9.91 |
| 27.262 | 2 | 22.61 | 45.00 | -22.39 | Vertical | 10.29 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.162 | 1 | 31.17 | 51.00 | -19.83 | Vertical | 9.91 |
| 27.2665 | 2 | 16.96 | 45.00 | -28.04 | Vertical | 10.29 |

Peak/Lim.Peak (2)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1.162 | 1 | 33.19 | 64.00 | -30.81 | Vertical | 9.91 |
| 27.262 | 2 | 22.61 | 58.00 | -35.39 | Vertical | 10.29 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz |
| --- | --- |

| name | 100kHz-10MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 10MHz-30MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 17/02/2025 19:22:27 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.908 | 1 | -1.22 | 4.00 | -5.22 | Vertical | 20.75 |
| 1175.908 | 2 | -1.80 | 4.00 | -5.80 | Horizontal | 20.75 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 17/02/2025 19:35:06 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1599.87325 | 1 | -0.66 | 4.00 | -4.66 | Vertical | 21.58 |
| 1599.8755 | 2 | -1.78 | 4.00 | -5.78 | Horizontal | 21.58 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 17:19:49 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.72 | 1 | 15.92 | 18.00 | -2.08 | Vertical | -14.23 |
| 164.25 | 2 | 17.89 | 18.00 | -0.11 | Horizontal | -14.62 |
| 432.02 | 3 | 6.12 | 18.00 | -11.88 | Vertical | -10.22 |
| 432.5 | 4 | 3.69 | 18.00 | -14.31 | Horizontal | -10.19 |
| 1971.65 | 5 | 14.68 | 30.00 | -15.32 | Vertical | 6.12 |
| 1972.28 | 6 | 14.61 | 30.00 | -15.39 | Horizontal | 6.12 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.72 | 1 | 21.15 | 25.00 | -3.85 | Vertical | -14.23 |
| 162.96 | 2 | 22.47 | 25.00 | -2.53 | Horizontal | -14.67 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.56 | 3 | 19.36 | 25.00 | -5.64 | Vertical | -10.19 |
| 434.27 | 4 | 18.07 | 25.00 | -6.93 | Horizontal | -10.11 |
| 1975.64 | 5 | 28.94 | 43.00 | -14.06 | Vertical | 6.15 |
| 1946.42 | 6 | 28.99 | 43.00 | -14.01 | Horizontal | 5.88 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 17:24:15 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 175.25 | 1 | 22.40 | 22.00 | 0.40 | Vertical | -14.21 |
| 172.25 | 2 | 20.98 | 22.00 | -1.02 | Horizontal | -14.32 |
| 514.5 | 3 | 32.23 | 28.00 | 4.23 | Vertical | -8.37 |
| 244.5 | 4 | 25.21 | 22.00 | 3.21 | Horizontal | -14.49 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 175.75 | 1 | 33.33 | 32.00 | 1.33 | Vertical | -14.19 |
| 172.75 | 2 | 31.68 | 32.00 | -0.32 | Horizontal | -14.30 |
| 238.25 | 3 | 37.64 | 32.00 | 5.64 | Vertical | -14.30 |
| 244.25 | 4 | 36.77 | 32.00 | 4.77 | Horizontal | -14.48 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of harness RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-03_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | VA/FK, 18/02/2025 17:38:02 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 96.42 | 1 | 17.50 | 12.00 | 5.50 | Vertical | -18.25 |
| 164.67 | 2 | 14.34 | 18.00 | -3.66 | Horizontal | -14.60 |
| 918.68 | 3 | 22.01 | 30.00 | -7.99 | Vertical | -2.65 |
| 432.44 | 4 | 5.55 | 18.00 | -12.45 | Horizontal | -10.20 |
| 1972.4 | 5 | 14.66 | 30.00 | -15.34 | Vertical | 6.13 |
| 1971.89 | 6 | 14.65 | 30.00 | -15.35 | Horizontal | 6.12 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 166.77 | 1 | 27.74 | 25.00 | 2.74 | Vertical | -14.52 |
| 163.41 | 2 | 18.99 | 25.00 | -6.01 | Horizontal | -14.65 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.68 | 3 | 20.94 | 25.00 | -4.06 | Vertical | -10.18 |
| 435.89 | 4 | 19.50 | 25.00 | -5.50 | Horizontal | -10.11 |
| 1992.92 | 5 | 28.95 | 43.00 | -14.05 | Vertical | 6.30 |
| 1960.4 | 6 | 28.75 | 43.00 | -14.25 | Horizontal | 6.02 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 17:41:19 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174 | 1 | 28.89 | 22.00 | 6.89 | Vertical | -14.25 |
| 171 | 2 | 18.57 | 22.00 | -3.43 | Horizontal | -14.36 |
| 244.25 | 3 | 33.58 | 22.00 | 11.58 | Vertical | -14.48 |
| 244.5 | 4 | 27.02 | 22.00 | 5.02 | Horizontal | -14.49 |
| 2137.5 | 5 | 27.19 | 36.00 | -8.81 | Vertical | 8.85 |
| 2467 | 6 | 26.98 | 36.00 | -9.02 | Horizontal | 9.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.5 | 1 | 38.50 | 32.00 | 6.50 | Vertical | -14.24 |
| 171.5 | 2 | 29.75 | 32.00 | -2.25 | Horizontal | -14.34 |
| 244 | 3 | 44.83 | 32.00 | 12.83 | Vertical | -14.47 |
| 244.75 | 4 | 38.48 | 32.00 | 6.48 | Horizontal | -14.50 |
| 2091.5 | 5 | 41.57 | 46.00 | -4.43 | Vertical | 8.83 |
| 2340.25 | 6 | 41.53 | 46.00 | -4.47 | Horizontal | 9.48 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 08:49:55 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Y, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.90575 | 1 | 0.16 | 4.00 | -3.84 | Vertical | 20.75 |
| 1175.90575 | 2 | 0.76 | 4.00 | -3.24 | Horizontal | 20.75 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 19/02/2025 15:51:50 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Y, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 87.51 | 1 | 19.41 | 12.00 | 7.41 | Vertical | -18.74 |
| 99.18 | 2 | 17.43 | 12.00 | 5.43 | Horizontal | -18.12 |
| 434.18 | 3 | 6.20 | 18.00 | -11.80 | Vertical | -10.11 |
| 432.02 | 4 | 5.55 | 18.00 | -12.45 | Horizontal | -10.22 |
| 1976.75 | 5 | 14.78 | 30.00 | -15.22 | Vertical | 6.16 |
| 1985.3 | 6 | 14.68 | 30.00 | -15.32 | Horizontal | 6.24 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 87.72 | 1 | 26.01 | 25.00 | 1.01 | Vertical | -18.72 |
| 155.91 | 2 | 26.91 | 25.00 | 1.91 | Horizontal | -14.99 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 434.69 | 3 | 20.18 | 25.00 | -4.82 | Vertical | -10.11 |
| 433.25 | 4 | 19.66 | 25.00 | -5.34 | Horizontal | -10.15 |
| 1984.64 | 5 | 28.82 | 43.00 | -14.18 | Vertical | 6.23 |
| 1966.67 | 6 | 28.72 | 43.00 | -14.28 | Horizontal | 6.08 |

Test parameters

| Name test: | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 19/02/2025 15:54:51 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Y, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 171 | 1 | 25.72 | 22.00 | 3.72 | Vertical | -14.36 |
| 187.25 | 2 | 26.85 | 22.00 | 4.85 | Horizontal | -13.64 |
| 236.75 | 3 | 33.23 | 22.00 | 11.23 | Vertical | -14.24 |
| 200 | 4 | 21.88 | 22.00 | -0.12 | Horizontal | -16.04 |
| 2137.5 | 5 | 29.71 | 36.00 | -6.29 | Vertical | 8.85 |
| 2137.5 | 6 | 27.07 | 36.00 | -8.93 | Horizontal | 8.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 172.5 | 1 | 35.70 | 32.00 | 3.70 | Vertical | -14.31 |
| 186.75 | 2 | 36.75 | 32.00 | 4.75 | Horizontal | -13.67 |
| 237 | 3 | 44.82 | 32.00 | 12.82 | Vertical | -14.25 |
| 239.25 | 4 | 33.77 | 32.00 | 1.77 | Horizontal | -14.35 |
| 2137.5 | 5 | 41.99 | 46.00 | -4.01 | Vertical | 8.85 |
| 2443 | 6 | 41.60 | 46.00 | -4.40 | Horizontal | 9.87 |

Test parameters

| Name test: | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 18/02/2025 09:05:59 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1175.90575 | 1 | 2.46 | 4.00 | -1.54 | Vertical | 20.75 |
| 1599.871 | 2 | -0.12 | 4.00 | -4.12 | Horizontal | 21.58 |

Test parameters

| Name test: | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.156GHz-1.617GHz |  | filename | CRE2-2025-TP002-03_ER_In front of DUT RBW 9kHz_GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | USE_CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 3 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 20/02/2025 10:57:23 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 87.78 | 1 | 19.52 | 12.00 | 7.52 | Vertical | -18.72 |
| 164.7 | 2 | 19.78 | 18.00 | 1.78 | Horizontal | -14.60 |
| 432.02 | 3 | 10.40 | 18.00 | -7.60 | Vertical | -10.22 |
| 432.56 | 4 | 9.26 | 18.00 | -8.74 | Horizontal | -10.19 |
| 1984.46 | 5 | 14.62 | 30.00 | -15.38 | Vertical | 6.23 |
| 1984.31 | 6 | 14.58 | 30.00 | -15.42 | Horizontal | 6.23 |

Q-Peak/Lim.Q-Peak (2)

| Frequency (MHz) | SR | Q-Peak (dBÂµV/m) | Lim.Q-Peak (dBÂµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 166.77 | 1 | 27.16 | 25.00 | 2.16 | Vertical | -14.52 |
| 164.28 | 2 | 24.12 | 25.00 | -0.88 | Horizontal | -14.61 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 432.29 | 3 | 22.93 | 25.00 | -2.07 | Vertical | -10.21 |
| 432.83 | 4 | 22.74 | 25.00 | -2.26 | Horizontal | -10.18 |
| 1967.57 | 5 | 28.97 | 43.00 | -14.03 | Vertical | 6.08 |
| 1976.18 | 6 | 29.18 | 43.00 | -13.82 | Horizontal | 6.16 |

Test parameters

| Name test: | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.805GHz-1.995GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

EQ/MR 01: Measurement of radio frequency radiated emission test

| Sample: |  |  | CRE2-2025-TP002-03 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | CRE2-2025-TP002 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 20/02/2025 10:13:20 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis Z, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 3, Conclusion : not comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBÂµV/m) | Lim.Avg (dBÂµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174 | 1 | 28.11 | 22.00 | 6.11 | Vertical | -14.25 |
| 171 | 2 | 24.31 | 22.00 | 2.31 | Horizontal | -14.36 |
| 243.75 | 3 | 31.51 | 22.00 | 9.51 | Vertical | -14.47 |
| 514.5 | 4 | 35.35 | 28.00 | 7.35 | Horizontal | -8.37 |
| 2137.5 | 5 | 27.95 | 36.00 | -8.05 | Vertical | 8.85 |
| 2467 | 6 | 26.98 | 36.00 | -9.02 | Horizontal | 9.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBÂµV/m) | Lim.Peak (dBÂµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 174.75 | 1 | 37.75 | 32.00 | 5.75 | Vertical | -14.23 |
| 174 | 2 | 34.35 | 32.00 | 2.35 | Horizontal | -14.25 |
| 244.5 | 3 | 42.79 | 32.00 | 10.79 | Vertical | -14.49 |
| 241 | 4 | 37.43 | 32.00 | 5.43 | Horizontal | -14.43 |
| 2491.5 | 5 | 41.34 | 46.00 | -4.66 | Vertical | 9.79 |
| 2444.5 | 6 | 41.29 | 46.00 | -4.71 | Horizontal | 9.87 |

Test parameters

| Name test: | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 171MHz-200MHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-2GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-6GHz |  | filename | CRE2-2025-TP002-02_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | USE_CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 3 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |