RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 16:46:11 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (6)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.5505 | 1 | 28.74 | 34.00 | -5.26 | Vertical | 9.85 |
| 13.78 | 2 | -2.89 | 18.00 | -20.89 | Vertical | 10.21 |
| 160.464 | 5 | -0.14 | 2.00 | -2.14 | Horizontal | 14.97 |
| 160.464 | 3 | -0.48 | 2.00 | -2.48 | Vertical | 14.97 |
| 433.028 | 4 | -9.48 | 14.00 | -23.48 | Vertical | -10.21 |
| 433.03025 | 6 | -9.92 | 14.00 | -23.92 | Horizontal | -10.21 |

Peak/Lim.Q-Peak (1)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Q-Peak (dBµV/m) | Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.5505 | 1 | 37.82 | 41.00 | -3.18 | Vertical | 9.85 |

Q-Peak/Lim.Q-Peak (1)

| Frequency (MHz) | SR | Q-Peak (dBµV/m) | Lim.Q-Peak (dBµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.5505 | 1 | 35.52 | 41.00 | -5.48 | Vertical | 9.85 |

Peak/Lim.Peak (6)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 0.33 | 1 | 45.78 | 62.00 | -16.22 | Vertical | 9.84 |
| 18.685 | 2 | 17.79 | 40.19 | -22.39 | Vertical | 10.22 |
| 30 | 3 | 2.04 | 39.00 | -36.96 | Vertical | 12.86 |
| 865.5005 | 4 | 10.70 | 40.00 | -29.30 | Vertical | -3.48 |
| 30 | 5 | 2.64 | 39.00 | -36.36 | Horizontal | 12.86 |
| 867.75275 | 6 | 10.14 | 40.00 | -29.86 | Horizontal | -3.48 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- |

| name | 150kHz-10MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 10MHz-30MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 9kHz-30MHz_rod |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-870MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-870MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 9kHz |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 16:26:25 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 1 | 3.22 | 12.00 | -8.78 | Vertical | -17.80 |
| 467.6 | 2 | 3.34 | 19.00 | -15.66 | Vertical | -8.59 |
| 119.97 | 3 | 3.26 | 12.00 | -8.74 | Horizontal | -17.80 |
| 465.86 | 4 | 3.38 | 19.00 | -15.62 | Horizontal | -8.53 |

Peak/Lim.Q-Peak (3)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Q-Peak (dBµV/m) | Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 627.95 | 2 | 23.22 | 58.00 | -34.78 | Vertical | -5.68 |
| 119.43 | 3 | 15.23 | 19.00 | -3.77 | Horizontal | -17.83 |
| 623.24 | 4 | 23.94 | 58.00 | -34.06 | Horizontal | -5.70 |

Q-Peak/Lim.Q-Peak (4)

| Frequency (MHz) | SR | Q-Peak (dBµV/m) | Lim.Q-Peak (dBµV/m) | Q-Peak-Lim.Q-Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 120 | 1 | 8.51 | 19.00 | -10.49 | Vertical | -17.80 |
| 627.77 | 2 | 17.17 | 58.00 | -40.83 | Vertical | -5.68 |
| 119.94 | 3 | 8.56 | 19.00 | -10.44 | Horizontal | -17.80 |
| 623.96 | 4 | 17.32 | 58.00 | -40.68 | Horizontal | -5.70 |

Peak/Lim.Peak (3)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 465.8 | 2 | 18.58 | 39.00 | -20.42 | Vertical | -8.53 |
| 54.63 | 3 | 19.48 | 57.00 | -37.52 | Horizontal | -19.48 |
| 461.33 | 4 | 18.22 | 39.00 | -20.78 | Horizontal | -8.81 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of harness RBW 120kHz |
| --- | --- |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 1 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 30MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 1 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-1GHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 120kHz |
| --- | --- | --- | --- | --- |
| step | 30kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 120kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 10000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 16:07:40 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 176.75 | 1 | 12.78 | 16.00 | -3.22 | Vertical | -14.76 |
| 241 | 2 | 13.03 | 16.00 | -2.97 | Vertical | -14.50 |
| 181 | 3 | 12.84 | 16.00 | -3.16 | Horizontal | -14.57 |
| 230.75 | 4 | 10.30 | 16.00 | -5.70 | Horizontal | -14.56 |

Peak/Lim.Peak (2)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 938.5 | 2 | 33.06 | 51.00 | -17.94 | Vertical | -2.87 |
| 941.25 | 4 | 32.04 | 51.00 | -18.96 | Horizontal | -2.80 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of harness RBW 1MHz |
| --- | --- |

| name | 170MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-960MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 170MHz-200MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 20MHz-300MHz_biconical with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 200MHz-960MHz |  | filename | CRE2-2025-TP020-07_ER_In front of harness RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 05/05/2025 07:50:50 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of harness, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | BGN ON, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1604.57225 | 1 | 7.34 | 20.00 | -12.66 | Vertical | 21.57 |
| 1604.57675 | 2 | 2.77 | 20.00 | -17.23 | Horizontal | 21.57 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 20:22:07 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1976.75 | 1 | 23.88 | 31.00 | -7.12 | Vertical | 6.13 |
| 2137.5 | 2 | 26.11 | 31.00 | -4.89 | Vertical | 8.81 |
| 1976.25 | 3 | 23.76 | 31.00 | -7.24 | Horizontal | 6.12 |
| 2137.5 | 4 | 26.00 | 31.00 | -5.00 | Horizontal | 8.81 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1977 | 1 | 37.48 | 51.00 | -13.52 | Vertical | 6.13 |
| 2138 | 2 | 40.18 | 51.00 | -10.82 | Vertical | 8.81 |
| 1981 | 3 | 38.11 | 51.00 | -12.89 | Horizontal | 6.16 |
| 2145.75 | 4 | 40.16 | 51.00 | -10.84 | Horizontal | 8.81 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 05/05/2025 07:56:45 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1604.58125 | 1 | 2.46 | 20.00 | -17.54 | Vertical | 21.57 |
| 1604.58125 | 2 | -1.23 | 20.00 | -21.23 | Horizontal | 21.57 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 20:27:19 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1976.75 | 1 | 23.85 | 31.00 | -7.15 | Vertical | 6.13 |
| 2137.5 | 2 | 26.40 | 31.00 | -4.60 | Vertical | 8.81 |
| 1976.75 | 3 | 23.76 | 31.00 | -7.24 | Horizontal | 6.13 |
| 2137.5 | 4 | 26.20 | 31.00 | -4.80 | Horizontal | 8.81 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1972.75 | 1 | 38.06 | 51.00 | -12.94 | Vertical | 6.09 |
| 2076 | 2 | 40.18 | 51.00 | -10.82 | Vertical | 8.74 |
| 1991 | 3 | 38.22 | 51.00 | -12.78 | Horizontal | 6.25 |
| 2147.25 | 4 | 40.11 | 51.00 | -10.89 | Horizontal | 8.81 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 05/05/2025 08:06:36 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (2)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1604.58125 | 1 | 6.93 | 20.00 | -13.07 | Vertical | 21.57 |
| 1604.579 | 2 | 6.57 | 20.00 | -13.43 | Horizontal | 21.57 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.553GHz-1.61GHz |  | filename | CRE2-2025-TP020-07_ER_in front of DUT 9kHz GNSS |
| --- | --- | --- | --- | --- |
| step | 2.25kHz |  | pre-amplifier | ON |
| set up | _CRE2_RESAC (A) 1.1-1.7GHz_GPS_horn |  | number of sweeping | 1 |
| RBW | 9kHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

RF emissions - Measuring using antennas (RE)

| Sample: |  |  | EMC-CRE2-2025-TP020-07 |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | EMC-CRE2-2025-TP020 RESAC |  |  |  |  |
| Operator: |  | NDN/WD, 02/05/2025 20:33:41 |  |  |  |
| Test Configuration: |  |  |  |  | Antenna position:in front of DUT, DUT Orientation:axis X, Housing connected to the ground plane:no, Configuration of the power return line LV:remotely grounded, - |
| Operating mode: |  |  |  | Mode 2, Conclusion : comply |  |

GraphiqueEMIRE

CISPR.AVG/Lim.Avg (4)

| Frequency (MHz) | SR | CISPR.AVG (dBµV/m) | Lim.Avg (dBµV/m) | CISPR.AVG-Lim.Avg (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1976.75 | 1 | 23.86 | 31.00 | -7.14 | Vertical | 6.13 |
| 2137.5 | 2 | 26.51 | 31.00 | -4.49 | Vertical | 8.81 |
| 1976 | 3 | 23.79 | 31.00 | -7.21 | Horizontal | 6.12 |
| 2137.5 | 4 | 26.02 | 31.00 | -4.98 | Horizontal | 8.81 |

Peak/Lim.Peak (4)

| Frequency (MHz) | SR | Peak (dBµV/m) | Lim.Peak (dBµV/m) | Peak-Lim.Peak (dB) | Polarization | Correction (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1976.25 | 1 | 38.17 | 51.00 | -12.83 | Vertical | 6.12 |
| 2100.25 | 2 | 40.68 | 51.00 | -10.32 | Vertical | 8.83 |
| 1986.75 | 3 | 37.76 | 51.00 | -13.24 | Horizontal | 6.21 |
| 2121.25 | 4 | 40.03 | 51.00 | -10.97 | Horizontal | 8.82 |

Test parameters

| Name test: | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 1.427GHz-2GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 200MHz-2GHz_log-periodic with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |

| name | 2GHz-5.925GHz |  | filename | CRE2-2025-TP020-07_ER_In front of DUT RBW 1MHz |
| --- | --- | --- | --- | --- |
| step | 250kHz |  | pre-amplifier | OFF |
| set up | _CRE2_RESAC (A) 1-7GHz_horn with preamp |  | number of sweeping | 1 |
| RBW | 1MHz |  | dynamic | 120 dB |
| Span | 0.1 MHz |  | reference level | 70 dBµV |
| VBW | Auto |  | RF attenuation | 0 dB |
| sweep time | 1000 ms/Step |  | RF attenuation min |  |
| Maxhold | 5 s |  |  |  |